Program for Calculation of Excitation Patterns

Brian R. Glasberg  (bg12@cus.cam.ac.uk) and Brian C.J. Moore  (bcjm@cam.ac.uk)

Department of Experimental Psychology, University of Cambridge,

Downing Street, Cambridge CB2 3EB, England

# I. INTRODUCTION

This document describes use of the executable program “excit2005”, which calculates excitation patterns according to the procedure described in Moore, Glasberg and Baer (1997), which was in turn modified from the procedure described by Glasberg and Moore (Glasberg and Moore, 1990).  The program should run on an IBM PC or compatible, using a “command prompt” or under “DOS”.

Each stimulus is specified in terms of its spectrum, which can be entered in several ways: as a series of sinusoidal components; as one or more pink or white noise bands; as a mixture of these two; or as the level in each successive one-third octave band.

# II. RUNNING THE PROGRAM

There are two ways to run the program excit2005:

A. Interactive mode

type

excit2005

and answer the questions one at a time.  The program can be interrupted at any time by typing “control c” (hold down the control key and type c).

B. Batch mode

The listening conditions and the spectrum of the stimulus are specified in an input file.  The name of the text file in which the results are stored is also specified in this file.  As an option, the user can specify the name of an output file that can be used directly with the “PLT” plotting program developed at the Boys Town Institute, Omaha, Nebraska.  This software can be obtained from:

http://www.boystownhospital.org/BasicClinic/neuro/cel/plt.asp

All characters in the input file must be plain text; there should be no special formatting characters.  The program excit2005 reads the input file, and saves the results in the output file.

As an example, type

excit2005 < example1.in

This creates a text output file with the calculated excitation pattern (example1.out) and also a file example1.plt which can be used with the PLT program.  The text output file (example1.out) has three columns, which show (from left to right), the centre frequency in Hz, the corresponding ERBN number (Glasberg and Moore, 1990; Moore, 2003), and the excitation level in dB.  The “reference” excitation level (0 dB) is the excitation produced by a 1000-Hz tone at 0 dB SPL when presented with frontal incidence in a free field (option f in the program).  When the excitation level is below –100 dB, the value is limited to –100 dB.

There are two “flags” (command line options); these are

–p	(the – denotes a “minus” sign)

This turns off some of the information that is printed to the screen.

–q

This prevents the “questions” from being printed to the screen. For example, type

excit2005  -p -q < example1.in

Note that if there is an existing file called example1.out , that file will be over-written without any warning being given.

# III. FORMAT OF INPUT FILE

The format of the input file varies depending on various options in the program; some of the questions asked by the program are conditional on the answers given to earlier questions.  Therefore, to be sure that an input file is in the right format, you may need to go through the stage of running the program in interactive mode with manual input.

A. Example input file

The contents of the example file example1.in are described below:

example1.out		Name of output file

example1.plt		Name of .plt file

lin                plot on linear frequency scale (omit if no plt file)

f                 	free-field presentation, frontal incidence

t                 	tonal input

i                 	inharmonic

1                 	number of components

1000.0         	frequency of component

40          		level of component

B. Layout of input file

If any line is left blank, the default value appropriate for that line is assumed, where appropriate.  However, for most lines a value must be specified.

The first two lines specify the names of the output files and plt files, respectively.   If a plt file name is given, then next line specifies whether the excitation pattern is to be plotted on a linear frequency scale (lin), a log scale (log) or an ERBN scale (erb).  When no plt file name is specified (the second line is left blank), this line is omitted.  The next line specifies the listening conditions.  Options are:

f 		free field with frontal incidence (default)

d		diffuse field

h 		headphones

If headphones are specified, the user has the option of naming a “correction file” which specifies the frequency response of the headphone at the eardrum.  If

n		no

is entered on the line following “h”, it is assumed that the headphones have a flat response at the eardrum, and that the sound levels specified are equivalent to levels at the eardrum.  If

y

is entered on the line following “h”, then the line after that must specify the name of the “correction file”.  For simplicity,  this should be located in the same directory as the executable program file excit2005.  An example of a correction file is “tdh.39”.  The first number in the file gives the number of frequencies at which the response is specified (30 in this example).  The maximum number of frequencies is 100.  The next four lines give the frequencies at which the response is specified.  The next four lines give the response at each frequency relative to the nominal sensitivity.  The text at the end of the file is a comment which is ignored by the program.

The next line indicates how the spectrum of the signal will be specified.  Options are:

t		complex tone, composed of discrete sinusoidal components

n		noise band(s), composed of white or pink noise

m		a mixture of a complex tone and one or more noise bands

o		levels to be specified in 26 1/3 octave bands covering centre frequencies from 50

to 16000 Hz

What comes next depends on what type of signal is specified at this stage.

If “t” is selected, then the next line specifies whether the signal is harmonic or inharmonic. Options are:

h		harmonic

i		inharmonic

The option “h” is useful for a complex tone that contains many harmonics.  If this option is selected, then the next line must specify the frequencies of the lowest harmonic, the highest harmonic and the fundamental frequency, separated by commas (all in Hz).  An example is

100, 1000, 1000

This specifies a tone with harmonics at 100, 200, 300, 400, 500, 600, 700, 800 900 and 1000 Hz.

The next line allows specific harmonics to be removed from an otherwise complete harmonic series.  The number of harmonics to be removed is initially specified.  For example:

1

indicates that one harmonic is to be removed.

If this line contains a number other than 0, then subsequent lines specify the frequencies of the harmonics to be removed (n lines for n removed components).

The next line allows additional components to be added, which do not need to be harmonically related to the complex tone specified earlier.  For example:

1

indicates that one frequency component is to be added.  If this line contains a number other than 0, the subsequent lines specify the frequencies of the components to be added (n lines for n added components).

The lines after this specify the level of each sinusoidal component in the input, first for the harmonic complex tone (in order of increasing frequency) and then for any added components (in the order specified).

That finishes the specification of a harmonic complex tone with removed/added components.

If the tone is specified as inharmonic (“i”), the next line indicates the number of sinusoidal components.  The subsequent lines indicate the frequency of each component (n lines for n components) and lines after that indicate the level of each frequency component, in the same order as the frequencies were specified.

If the signal is specified as one or more noise bands (“n”), the next line specifies the type of noise:

p		pink (spectrum level in passband decreases by 3 dB per doubling of frequency)

w		white (flat spectrum level within passband)

If “p” is specified, then the next line must give the frequency at which the spectrum level will be  specified.  The next line specifies the spectrum level, and the line after that specifies the lower and upper edge frequencies of the noise (in Hz), separated by a comma.

If “w” is specified, the next line specifies the number of noise bands.  For each noise band, subsequent lines specify the lower edge frequency,  the upper edge frequency and the spectrum level within the passband.

Following this initial specification of pink or white noise bands, there is an option for adding more pink or white noise bands.  If the next line contains

e		end

this indicates that no more noise bands are to be entered (end of noises) and the calculation then proceeds.

When the option “m” is selected (mixture of tones and noise), subsequent lines first specify the characteristics of the tonal part of the input, as described above, and second specify the characteristics of the noise part of the input, again as described above.

When the option “o” is selected, subsequent lines give the sound pressure levels in 26 adjacent 1/3 octave bands with nominal centre frequencies of 50, 62.5, 80, 100, 125, 160, 200, 250, 315, 400, 500, 630, 800, 1000, 1250, 1600, 2000, 2500, 3150, 4000, 5000, 6300, 8000, 10000, 12700 and 16000 Hz.  There should thus be 26 lines following the “o”.

Example input files provided with the software are:

example1.in	a simple 40 dB 1-kHz tone in free field

oct.in 		uses the 1/3 octave option described above (no plt output file)

pink.in		illustrates the use of a headphone correction file and a pink noise input

white.in		illustrates specification of two bands of white noise as the input

The example files have some comments in them.  However, to be sure that the program runs properly, it is best to avoid comments in the files.

Example headphone correction files are

tdh.39		tdh 39 headphones

hd580_05		Sennheiser HD580 headphones

# References

Glasberg, B. R., Moore, B. C. J., 1990. Derivation of auditory filter shapes from notched-noise data. Hear. Res. 47, 103-138.

Moore, B. C. J., Glasberg, B. R., Baer, T., 1997. A model for the prediction of thresholds, loudness and partial loudness. J. Audio Eng. Soc. 45, 224-240.

Moore, B. C. J., 2003. An Introduction to the Psychology of Hearing, 5th Ed., Academic Press, San Diego.

Please let us know if there are any problems, but no promises come with this software.