Department of Experimental Psychology, University of Cambridge,

Downing Street, Cambridge CB2 3EB, England.

This disc contains the executable program "LOUDAES" which should run on an

IBM PC or compatible. It calculates the loudness of a sound according to

the procedure described in Moore, Glasberg and Baer(1997).  This procedure

is only applicable to subjects with normal hearing.

Each stimulus is specified in terms of its spectrum, which can be

entered in several ways: as a series of sinusoidal components; as one or

more pink or white noise bands; as a mixture of these two; or as the

level in each successive one-third octave band.

The disc also contains the program "PARTLOUD" which can be used for

calculating the loudness of a sound in the presence of a background

sound.  This program is also applicable only to normal hearing.  For the

program PARTLOUD, the spectrum of the signal is entered separately from

that of the background.  If the signal is completely masked by the

background (according to the calculations performed by the program), a

message to that effect is given and no loudness is specified.

To run the program LOUDAES you can either just type

LOUDAES

and answer the questions one at a time (manual input), or you can use an

input file such as the example LOUDFF.IN and type

LOUDAES < LOUDFF.IN

There are two “flags” (command line options); these are

-p	This turns off some of the information that is printed to the

screen.

-q	This prevents the “questions” from being printed to the screen.

If you just want to know the final loudness, and you are using an input file,

use both options:

LOUDAES -p -q < LOUDFF.IN

By default, the answer is printed to the screen.  If you wish the result to be

stored in a file, type

LOUDAES < LOUDFF.IN > LOUDFF.OUT

The result will then be stored in the file LOUDFF.OUT

The format of the input file can vary depending on various options in

the program; some of the questions asked by the program are conditional

on the answers given to earlier questions.  Therefore, to be sure that

an input file is in the right format, you may need to go through the

stage of running the program with manual input.

The contents of the example file LOUDFF.IN are described below:

f		This selects free-field presentation (frontal incidence)

f		This selects frontal (as against diffuse) presentation

b		Specifies binaural presentation

m		This selects a mixed signal containing sinusoids and noise

h		This selects a harmonic signal for the sinusoidal part

800,1200,200	Frequencies of lowest harmonic, highest harmonic and F0

0		No components are removed (complete harmonic series)

1		One inharmonic component is added

1750		Frequency of added inharmonic component

60		Level in dB SPL of first harmonic component

65		Level in dB SPL of next harmonic component

63		Level in dB SPL of last harmonic component

61		Level of added inharmonic component

p		Pink noise is added

1500		Frequency at which the spectrum level of the pink noise

is specified

45		Spectrum level of the pink noise in dB at the specified

frequency

1000,1200	Lower and upper frequency limits of pink noise

20		Spacing of noise components; the noise is approximated as

a series of sinusoidal components with the specified

spacing.  For highest accuracy the spacing should be less

than 0.25 ERB in the frequency region of interest.  The

spacing can be increased to speed computation, but

to avoid large errors it should not be more than 0.5 ERB.

e		End of noise spectrum input

s		Stop execution

If an "s" is not encountered the program assumes that a new data set

will be present, i.e. it will calculate the loudness of the next signal

specified.  In this case it is assumed that the subject and conditions

of presentation are the same.  Thus, the program assumes that the

specification starts from line 4 (m above).

Note that the resulting output is the calculated total loudness of all

the sinusoidal components plus the noise.  The answer in this case is

14.48 sones, equivalent to 78.6 phons.

Another example input file is described below.  This file, LOUDHP.IN,

illustrates calculations for stimuli presented via headphones.

To run this type LOUDAES < LOUDHP.IN

The file contents are as follows

h		Specifies that headphone presentation is used.

y		A correction file is to be specified to allow for the

non-flat response of the headphone at the eardrum

tdh.39		The name of the correction file.  The first line in

this file specifies the number of points, N (28 in this

example). This is followed by a list of N frequencies,

separated by commas. Then follows a list of "corrections"

in dB, indicating the differences between the nominal input

level and the level actually achieved at the eardrum.

For example, for a nominal input level of 60 dB SPL, the

eardrum sound pressure at 80 Hz might be 44.7 dB SPL, so

the correction is 15.3 dB.  Note the convention that the

correction is positive when the eardrum SPL is below the

nominal SPL.

Some headphones (e.g. the Sennheiser HD414) are designed

to have a free-field response, i.e. their response at

the eardrum is similar to what would be achieved with

free-field presentation.  In such cases, it may be

simpler to specify f for free-field rather than h for

headphones.

b		Specifies binaural presentation

o		The stimulus will be specified as the levels in 1/3

octave bands.  The program runs more slowly when this

option is selected.

10		Level in first 1/3 octave band

10		Level in second 1/3 octave band

.

.

.

.

.

.

20		Level in 26th (last) 1/3 octave band

s		Stop execution

A third example input file is TONE.IN.

Try running it as:

LOUDAES -p -q < TONE.IN

The contents of this file are as

follows:

f		Free-field presentation

f		This selects frontal (as against diffuse) presentation

b		Binaural presentation

t		Tone signal

i		Inharmonic signal

1		One component

1000		Frequency of 1000 Hz

60		Level of 60 dB

t		Another tonal signal

i		Inharmonic signal

1		One component

1000		Frequency of 1000 Hz

70		Level of 70 dB

s		Stop execution

The program can be interrupted by typing ctrl-C (hold down the ctrl key

and press C).

The program PARTLOUD is used in a similar way.  You can either type

PARTLOUD

and answer the questions, or you can use an input file.  A suitable

input file is PARTLOUD.IN.   So, to run this program, type

PARTLOUD < PARTLOUD.IN

The contents of the input file are as follows:

f		This selects free-field presentation

f		This selects frontal incidence (not diffuse)

b		This selects binaural listening

t		This selects a complex tone signal

h		This selects a harmonic signal

800,1200,200	Frequencies of lowest harmonic, highest harmonic and F0

0		No components are removed (complete harmonic series)

1		One inharmonic component is added

1750		Frequency of added inharmonic component

60		Level in dB SPL of first harmonic component

65		Level in dB SPL of next harmonic component

63		Level in dB SPL of last harmonic component

61		Level of added inharmonic component

n		Type of partial masker - noise in this case

p		Pink noise for the partial masker

1500		Frequency at which the spectrum level of the pink noise

is specified

45		Spectrum level of the pink noise in dB at the specified

frequency

1000,1200	Lower and upper frequency limits of pink noise

20		Spacing of noise components

e		End of noise input

s		Stop execution

The calculated partial loudness is 7.95 sones, 69.7 phons.

Both programs give some information in addition to the calculated

loudness.  This information is suppressed when the -p flag is used.

The program LOUDAES displays four columns.  The first gives the

filter centre frequency (spaced uniformly in ERBs).  The second gives

the interpolated excitation threshold (in dB HL) at each frequency.  The

third gives the excitation level and the fourth gives the specific

loudness.  Results are only displayed for frequencies where the

excitation is above threshold.

The program PARTLOUD displays seven columns.  The first is an index

which should be ignored.  The second gives the number of ERBs. The third

gives the filter centre frequency (spaced uniformly in ERBs). The fourth

gives the total excitation level evoked by the signal and background.

The fifth gives the excitation level evoked by the background alone and

the sixth gives the excitation level evoked by the signal alone.  The

seventh gives the specific partial loudness.

Reference

Moore, B. C. J., Glasberg, B. R., and Baer, T. (1997). A model for the

prediction of thresholds, loudness and partial loudness. J. Audio Eng. Soc.,

45, 224-240.

Please let us know if there are any problems, but no promises

come with this software.

Brian Moore and Brian Glasberg, February, 1998

email: bcjm@cus.cam.ac.uk, bg12@cus.cam.ac.uk